# Unit 1: The Computing Environment | ENCO101

## Module Description

This module is intended to familiarise the students with the knowledge they require to develop competency and skills in the use of computing concepts required for the purposes of design, development and application within engineering settings.

### Unit Description

This unit provides an introduction to the computing environment in the context of engineering, focusing on the essential tools, software, and systems engineers use to solve complex problems. This unit emphasises how these elements integrate with engineering processes, where students will gain a comprehensive understanding of the hardware and software environments they will encounter in professional practice, along with an introduction to key programming languages used in the engineering industry.

## 1. Parameters that Affect Computer Hardware Choices

Engineers require hardware to software and work fast. Engineering software requires computers and laptops with ample random-access memory (RAM) to run on. Apple Mac has been the go-to brand for luxury and aesthetics, but in the engineering space, the more robust brands, such as DELL or Lenovo, are considered best to process software such as Autodesk, MATLAB and SOLIDWORKS, and programming software such as Python.

Engineers commonly consider the following key parameters when selecting the most suitable equipment for their needs:

- Central Processing Unit (CPU)

- Random Access Memory (RAM)

- Graphics Processing Unit (GPU)

- Storage (Hard Drives)

- Motherboard

- Power Supply Unit (PSU)

- Networking

### 1.1 Central Processing Unit (CPU)

The CPU is often referred to as the brain of the computer as it is the most critical component and performs all the processing tasks for the computer to function.

Central processing unit consists of:

- Clock Speed: Measured in gigahertz (GHz). It represents how fast the CPU can process instructions.

- Cores and Threads: Multi-core processes can handle multiple task simultaneously (think 101 tabs open). Hyper threading allows each core to work on multiple tasks.

- Cache Size: Larger caches can improve the performance by storing frequently accessed data. It’s the hardware of the software component that stores dates for future requests.

### 1.2 Random Access Memory (RAM)

RAM is a type of volatile computer memory that provides high speed access to the computer’s data that runs the applications. It’s called random access, because it can retrieve data quickly from any location in the memory, irrespective of where it is stored.

- Capacity: The amount of memory available to run applications. Higher capacity allows for better multitasking and handling for demanding applications.

- Speed: Measured in megahertz (MHZ) or gigahertz(GHz). It shows how fast data can be written or read.

### 1.3 Graphics Processing Unit (GPU)

GPU is a specialised electronic circuit that is designed to speed up the processing of images and videos for display:

- VRAM: Video Ram is the amount of memory of the graphics card (the memory for image display). This is more vital for gaming laptops.

### 1.4 Storage (Hard Drives)

Storage hard drives consists of the following:

- Capacity: the amount of storage a device can hold;

- Speed: For hard drives, the rotational speed (RPM) is crucial. SSDs, read and write speeds are important.

- Form Factor: The physical size and shape of the storage device.

### 1.5 Motherboard

A motherboard is the system board and serves as the central communication hub for various hardware components. It is a large circuit board that connects all the components together and includes:

- Form factor: Determines the size and shape of the motherboard

- Socket type: Expansion slots and ports

Figure 1-1: Image of a Motherboard

### 1.6 Power Supply Unit (PSU)

Power supply unit consists of the following:

- Wattage: Should be sufficient to power all components in the system with the same headroom for future upgrades.

- Efficiency Rating: Higher efficiency rating signifies better energy efficiency.

### 1.7 Networking

Networking consists of the following:

- Ethernet/wireless Adaptor: consider speed and wireless standard (Wi-Fi, 5G).

## 2. Basic Syntax

### 2.1 Data Types Definitions and Examples

Commands

In MATLAB, a command is a single line of code that performs a specific action. It is usually entered in the command window and executed immediately.

Example: disp(‘Hello, MATLAB!) is a command that displays the specific text.

Expression

An expression is a combination of numbers, variables, operators and functions, constants and operators that MATLAB evaluates to produce a result. It is always a value.

Example: a = 2 * (3 +4) is an expression that calculate the value of ‘a’ based on the given formula.

Variables

These are like placeholders or names. They can carry a multitude of data and information. They are used to store and manipulate data. They are created by assigning a value to a name.

Example: x = 5 creates the variable ‘x’, and assigns the value ‘5’.

Rules that variables have to abide by:

It must start with a letter.

It can contain letters, digits and the underscore character (_).

It cannot contain any punctuation (full stop, comma, semi-colon.), as these MATLAB syntaxes are already assigned to specific functions.

There shouldn’t be any spaces between characters When we have multiple variables that would use the same name, but have different characteristics, we use an underscore to show the difference between the data. For example, boy_height, boy_height, boy_name.

MATLAB is case-sensitive, which means a variable that uses a capital letter and a variable that uses a small letter do not mean the same thing.

One should avoid using the built-in functions in the variable since the program might not run the code.

String Operations

String operations in MATLAB involve manipulating and working with text data. MATLAB provides various functions for string manipulation, such as concatenation, extraction and conversion.

Example: strcat(‘Hello, ‘’, ‘MATLAB’) concatenates the three strings.

Arrays

Arrays in MATLAB are collections of elements with the same data type. MATLAB supports various types of arrays, including vectors and matrices and multi-dimensional arrays.

Example: A = [1 2 3 ;4 5 5]

Cells/Cell Arrays

These are data structures that can hold arrays of different types and sizes. Each cell can contain any type pf data, including other cells. ‘

Example: C = {1, ‘Hello’, [ 2 3 ;4 5]}

Structures

Structures in MATLAB allow you to group related data index a single name. Each piece of data is stored in a field.

Example: name = ‘John’, age = 25. This creates a structure with the fields ‘name’ and ‘age’.

Arguments:

Arguments are values or variables that are passed to a function when it is called. Functions use these arguments to perform operations and return results.

Example: sum ([1,2,3])’, the array ‘[1,2,3]’ is the argument.

Functions

Functions in MATLAB are blocks of code that perform a specific task. They take input arguments, perform operations, and return output values. MATLAB has built-in functions, and an option for users to create their own, custom functions.

Syntax

Syntax refers to the set of rules that dictates how MATLAB code should be written. It is vital to follow the correct syntax for the code to be interpreted and executed correctly.

Operations

Operations in MATLAB refer to the mathematical or logical operations performed on variables or the data. Examples include addition, subtraction, multiplication, division, and comparisons.

Script files

A script file is also known as a sequence of instruction commands carried out by MATLAB. These commands are known as programs. These script files are edited in the editor window and then displayed in the command window when executed or run. Script files have a ‘.m’ extension. How to run and save script files, can also change directory.

### 2.2 Arithmetic Operators with Scalars - Using MATLAB as a Calculator

Let’s discuss the standard arithmetic operations we constantly use with our calculators. These numbers can be assigned to variables:

| Operation | Symbol | Example |
| --- | --- | --- |
| Addition | + | 5+3 |
| Subtraction | - | 5-3 |
| Multiplication | * | 5*3 |
| Right division | / | 5/3 |
| Left division | \ | 5\3 = 3/5 |
| Exponentiation | ^ | 5^3 (means 53=125) |

Table 1-1: The Standard Operator and How It Would Be Written in MATLAB

### 2.3 Order of Precedence

MATLAB executes its calculations according to a specific order of precedence. This order is the same as the BODMAS (Bracket of Division, Multiplication, Addition, and Subtraction) rule learnt in Mathematics. Use the example below to indicate the MATLAB order of precedence

7+  2 x 3+4 2 -6

| Precedence | Mathematical operation | Example |
| --- | --- | --- |
| First | Bracket/ Parenthesis- these operators would follow the BRODMAS rule | 2 x 3+4- |
| Second | Exponents | (10 )2 =100 |
| Third | Multiplication, then division - this refers to that outside of the bracket | 7+ 100-6 |
| Fourth | Addition /subtraction | 101 |

Table 1-2: Order of Precedence

READ Additional Resources

Refer to the following resource in the Student Portal. Read: Prescribed Textbook, pages 13-16.

In MATLAB, the equal sign (=) is known as an assignment operator. It assigns or gives a specific value to a variable:

Variable _name = Numerical value / expression

HOW TO?

Example 3-1: clear;close all; clc; boy_height = 190; c = boy_height; c >> c = >> 190

Solution

In the editor window, type the variable: ‘boy_height = 190’. To execute the command in the command window, ensure that there is no semi-colon ‘;’, after the expression. The semi-colon prevents the expression from running in the line and only runs the expressions or codes after the semi-colon. To run the code, press ‘cntrl’ + ‘ENTER’ on your keyboard or the ‘run’ symbol in the banner. The code will show in the command window as above.

This becomes extremely useful if you are using the editor window to test-run codes independently.

### 2.4 Useful Commands

Using these commands can help you streamline your code when you move on to more complex coding and plots in MATLAB’ try these commands before executing all code. Type it before all code is written and separate it with semi-colons.

| Command | Output |
| --- | --- |
| clear | Deletes all variables from memory each time code is run. |
| Close all | Same as above. |
| clear x y z | Removes only variables specified, such as x, y, and z. |
| clc | Clears command window. |
| who | Displays a list of variables currently in the memory. |
| whos | Displays the above with other information such as bytes and class. |

### 2.5 Examples of MATLAB as a Calculator

HOW TO?

Example 3-2: A trigonometric identity is calculated by: 1-sinxcosx= cosx1+sinx Verify that the identity is correct on each side of the equation, and substitute x = π.

Solution

clear;close all; clc;

x = pi ;

LHS = (1-sin(x))/cos(x)

RHS = cos(x) / (1+ sin(x))

>>LHS =

-1.0000

>>RHS =

-1.0000

HOW TO?

Example 3-3 – Heat Transfer: A copper wire with an initial temperature, of T1, has a mass of 3kg and a specific heat capacity of 390 J/kg. ⁰C. and a final temperature of 40⁰. The copper is then submerged in a bucket of water with a mass of 6kg, an initial temperature of 20⁰C a final temperature of 23⁰ and a specific heat capacity of 4187 J/kg. ⁰C. Determine the initial temperature of the copper wire using the conservation of energy equation provided below: m1 x c1 x (T1 -T2 ) = m2 x c2 x (T2 -T3)

Solution

Before writing any script, you need to ensure that the equation is re-arranged so that T1 is unknown so that the equation can be solved. Once you have figured out the re-arrangement, you need to state the variables in the editor window and run the newly arranged equation.

clear;close all; clc;
m1= 3;
m2 = 6;
c1 = 390;
c2 = 4187;
t2 = 20;
t3 = 23;

t1 = ((m2 * c2 * (t2 - t3))/ ( m1 * c1)) + t2

>> t1 =

>> -44.4154

## 3. Arrays

### 3.1 Understanding Arrays

An array is a list of vector values that can be arranged in rows and/or columns. There are one-dimensional arrays which comprise of only columns or rows, and there are two-dimensional arrays which comprise of rows and columns. Arrays are the terms used to define a ‘matrix’.

HOW TO?

Example 4-1 –1D Arrays: Row Array: a= 68043 Column Array: b= 6 5251 3 x 3 Matrix: c= 223343111

### 3.2 How to Create Vectors from a Specified List or Table

First, you need to define the vector name and then, type it between square brackets and spaces as illustrated below:

Array_name = [ type the vector elements here]

HOW TO?

Example 4-2 – MATLAB Calculation: The following table shows data of rainfall per month from January until June. Use MATLAB to execute a calculation. 2023; Jan.; Feb.; March; April; May; June. Rainfall/month (in mm) - over 6 months.; 100; 40; 35; 20; 12; 20.

Solution

close all; clear; clc

rainfall_halfyear = [100 40 35 20 12 20]

>>rainfall_halfyear =

>>100    40    35    20    12    20

>> rainfall_halfyear =

100

40

35

20

12

20

Notice the difference when a semi-colon (;) is added between each vector. The semi-colon indicates that these vectors are in one column.

The ‘ENTER’ button can be used as an alternative to semi-colons between vectors. When using ‘ENTER’, it would line up the vectors below each other.

### 3.3 Vectors with Constant Spacing

Spacing in vectors are not what we actually think they are. Instead, the spacing refers to the increments between each vector. There are two ways in which we can create vectors with equal spacing.

Method 1

With brackets:

Variable _equal_spacing = [m: p:t]

Without brackets:

Variable _equal_spacing = m: p:t

The ‘m’, ‘p’, and ‘t’ are certain attributes in an array. The ‘m’ represents the first element and the ‘p’ represents the spacing between vectors. The ‘t’ represents the last element in the vector.

Method 2

We make use of the linspace function. When using this function, we state the first element, the last element and the number of elements in an array.

Variable_spacingmethod2 = linspace (xi, xf, n)

close all;clear;clc

v1 = linspace (20, 60, 10)

>>v1 =

Columns 1 through 7

20.0000   24.4444   28.8889   33.3333   37.7778   42.2222   46.6667

Columns 8 through 10

51.1111   55.5556   60.0000

close all;clear;clc

v2 = linspace (-4, -50, 12)

>>v2 =

Columns 1 through 7

-4.0000   -8.1818 -12.3636  -16.5455  -20.7273  -24.9091   -29.0909

Columns 8 through 12

-33.2727 -37.4545  -41.6364  -45.8182  -50.0000

### 3.4 Two-Dimensional Arrays – Matrices

Matrices comprise of rows and columns, read in that order.

a = i1 j1i1 j2i1 j3i2 j1i2 j2i2 j3i3 j1i3 j2i3 j3

The ‘i’ represents the row number and the ‘j’ represents the column number. For example, ‘i1j3’, is row 1, column 3. A matrix can be created in MATLAB by assigning elements to a variable inside square brackets, [ ]. To create multiple rows, the elements have to be separated by a semi-colon (;).

The other method used in making arrays can also be used to make matrices; instead of separating the elements in each row with a semi-colon, we can press ‘ENTER’ between each row so that the new row sits on a new line. This instruction tells MATLAB that this is a new row in the matrix.

close all;clear;clc

a = [1,3,4 ; 2,0,0 ;3,1,1]

>> a =

1     3     4

close all;clear;clc

a = [1,3,4

2,0,0

3,1,1]

>> a =

1     3     4
     2     0     0
     3     1     1

### 3.5 Matrix Function in MATLAB

We can translate matrix functions or types into MATLAB. When we for instance want a matrix with only zeros, only ones, or ones in the diagonal, we make use of built-in functions with MATLAB to create these.

To create a zero matrix we use the zeros(m,n) fiction. If we want only ones in the matrix, we use the ones(m,n) functions. And if we want certain values, like only ones, down the diagonal, we use the eye(n) function. The m-element stands for rows, and the n-elements stands for columns.

close all;clear;clc

a = zeros (4,3)

b = eye (4,3)

c = ones (3,4)

>> a =

0     0     0

0     0     0

0     0     0

0     0     0

>> b =

1     0     0

0     1     0

0     0     1

0     0     0

>> c =

1     1     1     1

1     1     1     1

1     1     1     1

If we have a closer look at variables, there are a few attributes we must consider. All variables are arrays. A scalar variable has one element, and a vector variable has rows and columns. Although we define a size for specific vector arrays and matrices, for scalar variables this is not needed.

When we assign a variable to an element, whether it be a scalar or vector, it can be changed to any other type or size of variables by deleting or added the necessary elements.

### 3.6 Transpose of a Matrix

The transpose of a matrix, when applied to vectors, converts columns to rows and rows to columns. To create a transposed matrix, we first assign element to a variable in matrix format; and then a new variable is created based on the assigned matrix variable. The difference is that the previous variable now has an apostrophe (‘) after it.

close all; clear;clc

a1 = [2 3 6 8 0 0 ]

A = a1'

b1 = [ 2 4 6 7 ; 3 4 6 7 ; 4 6 7 0 ; 0 0 3 1]

B= b1'

>> a1 =

2     3     6     8     0     0

>> A =

2

3

6

8

0

0

7     7     0     1

>> b1 =

2     4     6     7

3     4     6     7

4     6     7     0

0     0     3     1

>> B =

2     3     4     0

4     4     6     0

6     6     7     3

7     7     0     1

### 3.7 Attributes of Matrices, Vectors and Arrays

The position of an element in an array, vector or matrix can be seen as a school bag in pigeon or cubby holes. We take out certain backpacks (elements) in certain cubby holes (rows and columns) and manipulate this data for further use. When we retrieve or address this specific cubby hole (rows and column), we do not necessarily want the data, instead we want the data in that specific position to be used for something further on.

We can do the following to vectors and matrices:

Address or retrieve elements individually or in subgroups

Create new vectors or matrices from another

Add elements of existing variables

Delete elements

Use built in functions to change the properties of matrices and vectors

Addressing arrays and matrices are done differently due to their nature. When we have an array, it can either be in column or row format. When we address a position of a specific element, one needs to bear in mind that the first element will be addressed by number 1 and not 0, as is done in Python. We determined the corresponding element by placing our instruction in the command window.

We used the following format for arrays:

Variable_box_name_ = [ x r t y t d];

Variable_box_name_ (k)

And we have the corresponding single vectors:

Variable_box_name_ (3) = t

Variable_box_name_ (6) = d

In longer code, we can manipulate the data in that position by addressing that specific element, and then assigning it a known value. The data can then be manipulated by adding and subtracting those elements or using built-in functions. We can do the same with matrices with the following method below:

MatrixA = i1 j1i1 j2i1 j3i2 j1i2 j2i2 j3i3 j1i3 j2i3 j3 and ,

MatrixA(m, n)

close all; clear; clc

% Addressing elements

Vector = [ 3 4 7 0 0 8 6 4 2 3]

Vector (6)

Vector (3)

% Assigning a new value

Vector (4) = 45

% Mathematical Operations

Vector (3) + Vector (7)

Vector (5) ^Vector (1) + sqrt (Vector (2))

>> Vector =

3     4     7     0     0     8     6     4     2     3

>> ans =     8

>>ans =     7

M = [ 2 6 9 5; 10 11 6 9; 10 13 6 5]

M(2,2)

>> M =

2     6     9     5

10    11     6     9

10    13     6     5

>> ans = 11

>> M =

2     6     9     5

100    11     6     9

10    13     6     5

M = [ 2 6 9 5; 10 11 6 9; 10 13 6 5]

M (2,2) - M (3,1)

The above tutorial shows how we can access elements in matrices and use these elements for data manipulation. A colon can be utilised to address elements in a vector or matrix.

| Command | Explanation |
| --- | --- |
| V1(:) | Refers to accessing all the elements in that vector |
| V1(p:q) | Accessing vectors from p until and including q |
| M(:, q) | All elements in ALL rows of column n |
| M( p, : ) | All elements in all columns of row p |
| M ( :, p :q) | All elements in all columns between rows p and q |
| M (m :n, p :q) | Elements in rows m through n and columns p through q |

Table 4-1: How to access Matrices

clc; close all; clear ;

% Creating a Matrix
M = [ 5 2 6 9 12 ; 2 9 1 15 18 ; 5 9 0 0 11; 17 13 16 2 9; 10 12 16 3 9 ]

% Accessing column 5
S = M(:,5)

% Accessing row 2 and 3
T = M(2:3 ,:)

% Accessing column 2 & 3 , from row 1 through 4
U = M(1:4, 2:3)

>> M =

5     2     6     9    12
     2     9     1    15    18
     5     9     0     0    11
    17    13    16     2     9
    10    12    16     3     9

>> S =
    12
    18
    11

>> T =
     2     9     1    15    18
     5     9     0     0    11

>> U =
     2     6
     9     1
     9     0
    13    16

% Creating a vector from value 6 , in increments of 3 , until 30

v = [6 : 3: 30]

% creating a new vector from the previous one
% creating a new vector from the 4th, 6th and 9th element until element 10

u = v([4,6,7 : 9])

>> v =
     6     9    12    15    18    21    24    27    30

>>

R = [10: -3: -6; ones(1,6); 2:2:12; zeros(1,6)]

>> R =

10     7     4     1    -2    -5

1     1     1     1     1     1

2     4     6     8    10    12

0     0     0     0     0     0

% Create matrix Z from Matrix R

Z = R ([1,4],[1,3,6 : 6])

>> Z =

10     4    -5
     0     0     0

### 3.8 Adding and Deleting Elements in Arrays and Matrices

We can add and remove elements in matrices and arrays. We can add extra rows and columns to an existing matrix. The reason for using these functions is that when we get to loops, these functions are needed-depending on what we intend to do with them. There are additional built-in functions when deleting and adding elements and rows.

There are multiple ways of adding and deleting elements in both arrays and matrices.

clc, clear all

% First define a new array

DF = 1:4

>> DF =      1     2     3     4

% We can now extend it by adding on elements.

% What now added 5 new columns, starting at 10 and ending at 0.

% If we said 40 instead of 35, note that it would be undefined since there

is only supposed to be 5 columns.

DF(5:10) = 10:5:35

>> DF =     1     2     3     4     10     15     20     25     3-     35

AF= [ 4 7 9]

% State that we want column 10 (which is new) to have the value of 8.

AD(10) = 8

>> AF =     4     7     9

>> AD =     0     0     0     0     0     0     0     0     0     8

We can also define new vectors by appending existing vectors into a new vector.

AX = [ 5 8 6 20]

CV = 4:3:16

% Define a new vector, we need to transpose it since it won’t be able to fit

% if they are both row arrays

KN = [AX'; CV']

>> AX =

5     8     6     20

>> CV =

4     7     10     13     16

>> KN = 5

8

6

20

4

7

10

13

16

It is a bit different for matrices.

Let’s have a closer look below.

clc, clear all

% Let's create a matrix

A = [1 2 3 4 5;5 6 4 2 2]

A (3,:) = [10:5:30]

% you need to know the increments and end of the matrix

A(3,5) = 100

% place the value 100 in the element row3, column5

B = eye(3)

C = [A B]

>> A =  1     2     3     4     5

5     6     4     2     2

>> A =  1     2     3     4     5

5     6     4     2     2

10    15    20    25    30

>> A =  1     2     3     4     5

5     6     4     2     2

10    15    20    25   100

>> B =  1     0     0

0     1     0

0     0     1

>> C =  1     2     3     4     5     1     0     0

5     6     4     2     2     0     1     0

10    15    20    25   100     0     0     1

### 3.9 Strings and Variables

Strings and variables consists of the following:

A string is an array of characters.

Strings are created by characters’ sin single quote marks.

It can include; better, digits, symbols etc., e.g. ‘kjdjj’, ‘3%$$*’.

They have multiple functions:

Display text messages

Format commands or plots

MATLAB has a built-in function named char that creates an array with rows containing the same number of characters. Another way to lock at strings is to view it as changing numbers to words. Any string is converted to its original function, to having ‘no function’.

Variable_name = char (‘string1’, string 2’, string 3’)

clc, clear all

Variable1 ='My name is blah blah'

Variable2 = ' 1+ 2 = 2, but 2+2 = 4'

Student_info = char('Name', 'Surname', 'Grade', 'Subjects')

>> Variable1 =

'My name is blah blah'

>> Variable2 =

' 1+ 2 = 2, but 2+2 = 4'

>> Student_info =

4×8 char array

'Name    '
    'Surname '
    'Grade   '
    'Subjects'