# Activity 5-2

### Question 1

A shaft that is rotating at 200 rev/min is uniformly decelerated to rest in 30 seconds.

Determine the number of revolutions it makes during this time.

### Question 2

A grinding wheel takes 100 revolutions to decelerate uniformly from 500 r/min to 350 r/min.

Calculate its angular acceleration.

### Question 3

A grinding wheel has an angular deceleration of 1,3 rad/s2.

How many revolutions does it take for it to slow uniformly from 700 r/min to 200 r/min?

### Question 4

A flywheel is accelerated uniformly from rest to 2000 r/min in two minutes.

Calculate its angular acceleration and the number of revolutions it turns:

- in the first 45 seconds.

- up to the moment when it reaches 1400 r/min.

### Question 5

A generator is driven by a belt passing around a 150 mm diameter pulley. The pulley is rotating at 2500 rev/min and is brought to rest in 175 revolutions by a uniform retardation.

Calculate the angular retardation of the generator shaft and the linear deceleration of the pulley belt assuming there is no slip.

### Question 6

A motorcycle’s wheels are 520 mm in diameter and accelerates uniformly from 36 km/hr to 108 km/hr while covering a distance of 200 m. Calculate:

- the angular acceleration of the wheels.

- the number of revolutions made by the wheels during the period that the motorcycle is accelerating.

### Question 7

A flywheel starts from rest and accelerates uniformly at 2 rad/sec2 until it reaches a speed of 500 r/min. It maintains this speed for a certain period after which it comes to rest with a constant deceleration of 5 rad/sec2.

A total of 600 revolutions were made by the flywheel. Draw an angular velocity–time graph and use it to determine the total time taken from start to stop.

### Question 8

A shaft starts from rest and accelerates uniformly while turning through 200 revolutions, reaching a speed of 1500 r/min. This speed is maintained until the shaft decelerates and stops after turning through a total of 800 revolutions.

Assume that the total time taken is 50 sec. Draw an angular velocity-time graph and use it to find the magnitude of the angular retardation.