# Activity 6-1

### Question 1

A bus with a mass of 4700 kg must travel at a constant speed of 38 km/hr over a distance of 200 m up an incline of 1 in 50. The tractive resistance of the bus is 150 N per tonne of its mass. Calculate the engine power required, assuming an overall efficiency of 82%.

### Question 2

400 kW of power is supplied to the motor pulling a train at 50 km/hr up a slope of 1:200. Tractive resistance is 75 N/tonne of the train’s mass. Assume the efficiency of the motor is 90% and calculate the mass of the train.

### Question 3

A 200 tonne locomotive develops a maximum power of 400 kW.

The tractive resistance to motion is 2% of the weight of the train. Calculate the maximum speed in km/hr at which the locomotive can pull the train down a slope of 1 in 300.

### Question 4

A lift of weight 20 kN is raised a vertical distance of 300 m in 4 minutes at constant velocity by a cable of mass 7 kg/m. The cable is wound around a drum mechanism driven by an electric motor having an efficiency of 68%.

Calculate the work done in raising the lift from a depth of 200 m to a depth of 70 m and determine the average power necessary to raise the lift from the 300 m level to the surface.

### Question 5

An electric motor causes a light 800 mm diameter drum to rotate at 50 rev/min and to lift a 100 kg mass by means of a light rope.

The torque required to overcome friction is 20 N.m. Calculate the motor power required, assuming an efficiency of 92%.

### Question 6

A shaft has a diameter of 100 mm and rests between two bearings 1 m apart. The shaft carries a load of 75 kN acting between the bearings and is 250 mm from one of them. The coefficient of friction is 0,03 in the bearing nearest to the load and 0,04 in the other.

Calculate the power required to overcome friction at 200 rev/min.