# Activity 6-2

### Question 1

An 800 kg car accelerating down a 1 in 60 slope at 2 m/s2 experiences a tractive resistance of         400 N.

Calculate the magnitude of the tractive effort required and the engine power at 70 km/hr if the overall efficiency is 60%.

### Question 2

A motorcycle is travelling up a slope of 1 in 50 at a speed of 120 km/hr when its engine dies. The tractive resistance of the motorcycle is equal to 20% of its weight.

Calculate the distance the motorcycle free-wheels up the hill before coming to rest.

### Question 3

A train of mass 270 tones has a tractive resistance of 0, 8% of its weight. The train reaches a speed of 90 km/hr from rest in a distance of 1 km up a 1 in 120 slope.

Assuming uniform acceleration, calculate the required tractive effort and the motor power at the instant the train is moving at 80 km/hr. Assume an overall efficiency of 66%.

### Question 4

A squash ball of mass 50 grams hits a wall at 20 m/s and bounces back at 8 m/s.

If the ball is in contact with the wall for 0, 07 seconds, find the average force against the wall.

### Question 5

A cannon weighing 490 kN fires a 200 kg shell with a muzzle velocity of 500 m/s.

Calculate the initial recoil velocity of the cannon and determine the magnitude of the constant force required to bring the cannon to rest in a distance of 1, 5 m.

### Question 6

A helical spring has a stiffness of 5 kN/m and a free length of 450 mm. The spring is compressed to a length of 340 mm and a 0,5 kg mass is placed on it.

Calculate the maximum height reached by the mass when the spring is suddenly released.